Table of Contents

# OpenWrt Captive Portal with Email OTP Authentication

## Technical Implementation Report

Project: Email-Based OTP Captive Portal for OpenWrt Implementation: Custom Firewall + Flask Authentication Server Date: November 2024 Status: Complete and Deployed

## Executive Summary

This project implements a complete captive portal solution for OpenWrt routers using email-based One-Time Password (OTP) authentication. The system consists of a Flask-based authentication server running on a host machine and custom firewall rules on the OpenWrt router that work together to provide secure WiFi access control.

### Key Features

- Email-Based Authentication: Users receive 6-digit OTP codes via email

- RFC 8910 Compliant: Proper captive portal detection for all major platforms

- Secure Firewall: Custom iptables chains for granular access control

- Session Management: Time-based sessions with automatic expiration

- Real-Time Dashboard: Web-based admin interface for monitoring

- Multi-Platform Support: Works with iOS, Android, Windows, macOS, and Linux

## System Architecture

### High-Level Overview

┌─────────────┐         ┌──────────────┐         ┌─────────────────┐
│   Client    │◄───────►│   OpenWrt    │◄───────►│  OTP Auth       │
│   Device    │  WiFi   │   Router     │  HTTP   │  Server (Host)  │
└─────────────┘         └──────────────┘         └─────────────────┘
                              │                           │
                              │                           │
                         iptables                    Flask + SMTP
                         Firewall                    Email Service

### Network Topology

- Router LAN Interface: eth1 (10.0.10.1/24)

- Router WAN Interface: eth0

- Host-Router Bridge: eth2 (192.168.56.0/24)

- OTP Server: 192.168.56.1:5000

- DNS Hijacking: Selective (captive portal detection URLs only)

## Component Breakdown

### 1. Authentication Server (Flask Application)

File: otp_auth_server.py Technology: Python 3, Flask, SMTP Port: 5000

#### Core Functionality

OTP Generation and Management - Generates cryptographically secure 6-digit OTPs - Stores active OTPs in memory with expiration tracking - Validates OTP uniqueness - Automatic cleanup of expired codes (5-minute validity)

Email Delivery - HTML-formatted email templates with embedded OTP - Plain text fallback for compatibility - SMTP with TLS encryption - Configurable email providers (tested with Disroot)

Session Management - Token-based authentication (32-byte URL-safe tokens) - MAC address binding - Configurable session duration (default: 1 hour) - Automatic session expiration

API Endpoints

| Endpoint | Method | Purpose |
| --- | --- | --- |
| /api/request_otp | POST | Request OTP code via email |
| /api/verify_otp | POST/GET | Verify OTP and authenticate |
| /api/check_auth | GET/POST | Check authentication status |
| /api/stats | GET | Get server statistics |
| / | GET | Admin dashboard (HTML) |

#### Configuration Parameters

OTP_LENGTH = 6              # OTP digit count
OTP_VALIDITY = 300          # 5 minutes
SESSION_DURATION = 3600     # 1 hour
EMAIL_ENABLED = False       # Test mode by default
SMTP_SERVER = "smtp.example.com"
SMTP_PORT = 587

### 2. Router Firewall Configuration

File: router/etc/firewall.captive Technology: Bash script with iptables Execution: Auto-run on boot via rc.local

#### Firewall Chain Architecture

FORWARD Chain
     │
     ├──► CAPTIVE_PORTAL
              │
              ├──► CAPTIVE_ACCEPT (authenticated clients)
              │         └──► ACCEPT (all traffic)
              │
              ├──► CAPTIVE_DNS (DNS queries)
              │         ├──► Router DNS → ACCEPT
              │         └──► External DNS → REJECT
              │
              ├──► Router Traffic (10.0.10.1) → ACCEPT
              │
              └──► All Other → REJECT

#### Custom iptables Chains

CAPTIVE_PORTAL Chain - Main entry point for client traffic - Routes to appropriate sub-chains - Blocks all traffic except router and authenticated clients

CAPTIVE_ACCEPT Chain - Contains MAC-based ACCEPT rules for authenticated clients - Rules inserted at position 1 for priority - Checked before other rules

CAPTIVE_DNS Chain - Allows DNS queries to router only (10.0.10.1) - Blocks external DNS for unauthenticated clients - Authenticated clients get DNS redirect to 8.8.8.8

#### NAT Rules

HTTP Redirect (PREROUTING)

iptables -t nat -I PREROUTING 1 -i eth1 -p tcp --dport 80 \
    -j DNAT --to-destination 10.0.10.1:80

- Redirects all HTTP traffic to splash page

- Authenticated clients get RETURN rules to bypass

OTP Server Forwarding

iptables -t nat -A PREROUTING -i eth1 -p tcp --dport 8080 \
    -j DNAT --to-destination 192.168.56.1:5000

- Forwards port 8080 to OTP server on host

Masquerading

iptables -t nat -A POSTROUTING -o eth0 -j MASQUERADE
iptables -t nat -A POSTROUTING -p tcp -d 192.168.56.1 \
    --dport 5000 -j MASQUERADE

### 3. DNS Configuration (dnsmasq)

#### Selective DNS Hijacking (RFC 8910 Compliant)

Instead of hijacking all DNS queries, only captive portal detection URLs are redirected:

address=/captive.apple.com/10.0.10.1
address=/connectivitycheck.gstatic.com/10.0.10.1
address=/detectportal.firefox.com/10.0.10.1
address=/www.msftconnecttest.com/10.0.10.1
address=/clients3.google.com/10.0.10.1

#### DHCP Options for Native Portal Detection

dhcp-option=114,http://10.0.10.1/simple-otp.html
dhcp-option=160,http://10.0.10.1/cgi-bin/captive-detect

Benefits: - Native browser notifications - Automatic portal popup on iOS/Android - Better user experience - Standards-compliant

### 4. CGI Scripts (Router Web Interface)

Location: /www/cgi-bin/ Server: uhttpd (OpenWrt default)

#### Authentication Endpoint (auth)

URL: http://10.0.10.1/cgi-bin/auth Purpose: Receive authentication requests from OTP server

Parameters: - action: auth/deauth/list - mac: Client MAC address - ip: Client IP address (optional)

Response: JSON with status and message

Example:

curl "http://10.0.10.1/cgi-bin/auth?action=auth&mac=AA:BB:CC:DD:EE:FF&ip=10.0.10.50"

#### MAC Detection Endpoint (get-mac)

Purpose: Detect client MAC from IP using ARP table Method: Reads /proc/net/arp

Response:

{
  "success": true,
  "mac": "AA:BB:CC:DD:EE:FF",
  "ip": "10.0.10.50"
}

#### API Proxy (api-proxy)

Purpose: Forward API requests to OTP server Why: Avoid mixed content warnings (HTTP/HTTPS)

Forwards: - POST requests with JSON body - GET requests with query parameters - Sets CORS headers

#### Captive Portal Detection Handler (captive-detect)

Purpose: Smart responses based on authentication status

Authenticated Clients: - Apple: Returns “Success” - Android: Returns HTTP 204 - Firefox: Returns “success” - Windows: Returns “Microsoft Connect Test”

Unauthenticated Clients: - HTTP 302 redirect to splash page - Works with all platform detection methods

### 5. Authentication Binary

File: router/usr/bin/captive-auth Language: Shell script Purpose: Manage client authentication state

#### Commands

Authenticate Client

captive-auth auth AA:BB:CC:DD:EE:FF 10.0.10.50

Actions Performed: 1. Add MAC to CAPTIVE_ACCEPT chain → Internet access 2. Add MAC to CAPTIVE_DNS chain → DNS bypass 3. Add NAT RETURN rule → HTTP redirect bypass 4. Redirect DNS to 8.8.8.8 → Real DNS queries

De-authenticate Client

captive-auth deauth AA:BB:CC:DD:EE:FF 10.0.10.50

List Authenticated Clients

captive-auth list

### 6. Splash Pages

#### Main Splash Page (splash_otp.html)

Features: - Modern, responsive design - 3-step authentication flow: 1. Email entry 2. OTP code entry (6 individual input boxes) 3. Success confirmation - Client-side validation - Real-time feedback messages - Loading indicators - Auto-focus and auto-advance inputs - Paste support for OTP codes

Technical Details: - Pure JavaScript (no frameworks) - Fetch API for HTTP requests - CSS gradients and animations - Mobile-friendly (viewport meta tag) - Cache prevention headers

User Flow:

User Connects → Redirect → Email Entry → Email Sent
     ↓
OTP Entry → Verification → Success → Internet Access

#### Router Splash Page (simple-otp.html)

Simplified version served directly from router for detection endpoints.

## Authentication Flow (Detailed)

### Step-by-Step Process

1. Client Connects to WiFi - DHCP assigns IP (10.0.10.x) - Receives DHCP option 114 (captive portal URL) - DNS configured to router (10.0.10.1)

2. Captive Portal Detection - Device attempts connectivity check - DNS query for detection URL (e.g., captive.apple.com) - dnsmasq returns router IP (10.0.10.1) - Browser/OS detects captive portal

3. HTTP Redirect - User attempts to browse (HTTP request) - iptables NAT PREROUTING redirects to router - Splash page served from /www/simple-otp.html

4. Email Submission - User enters email address - JavaScript validates format - POST to http://10.0.10.1:8080/api/request_otp - Forwarded to OTP server (192.168.56.1:5000)

5. OTP Generation - Server generates 6-digit OTP - Stores in active_otps dictionary - Sends HTML email via SMTP - Returns success to client

6. OTP Verification - User receives email with OTP code - Enters 6 digits in splash page - JavaScript sends MAC + OTP to server - Server validates OTP and marks as used

7. Router Authentication - OTP server calls router auth endpoint - Router executes captive-auth auth MAC IP - iptables rules added for client - Client gains internet access

8. Confirmation - Client receives success response - Splash page shows success message - Browsers recheck connectivity - Portal popup closes automatically

## Security Considerations

### Implemented Security Measures

1. OTP Security - Cryptographically secure random generation (secrets module) - Single-use codes (marked as used after verification) - Time-based expiration (5 minutes) - Unique OTP validation before storage

2. Session Security - Token-based authentication (32-byte tokens) - MAC address binding - Session expiration tracking - Automatic cleanup of expired sessions

3. Network Security - Default REJECT policy for unauthenticated clients - HTTPS traffic blocked (forces detection) - DNS queries restricted to router only - Selective DNS hijacking (not all domains)

4. Email Security - TLS encryption for SMTP (STARTTLS) - HTML email with proper formatting - Plain text fallback

5. Input Validation - Email format validation (regex) - MAC address format checking - OTP format verification (6 digits) - SQL injection prevention (no database used)

### Potential Vulnerabilities and Mitigations

MAC Address Spoofing - Risk: Attacker could spoof authenticated MAC - Mitigation: Short session durations, monitoring logs - Future: Add IP+MAC binding, timeout on inactivity

Man-in-the-Middle - Risk: HTTP traffic is unencrypted - Mitigation: Use HTTPS for OTP server, local network isolation - Future: Implement SSL/TLS on router

Denial of Service - Risk: OTP request flooding - Mitigation: Rate limiting per email/IP needed - Future: Add CAPTCHA, request throttling

Email Interception - Risk: OTP sent over email could be intercepted - Mitigation: Short OTP validity, SMTP TLS - Future: Add SMS option, 2FA

## Deployment Guide

### Prerequisites

Router Requirements: - OpenWrt 24.10+ or compatible - Minimum 128MB RAM - Network interfaces: eth0 (WAN), eth1 (LAN), eth2 (host bridge) - Packages: iptables, dnsmasq, uhttpd

Host Requirements: - Python 3.7+ - Packages: flask, flask-cors, requests - Network connectivity to router - Email account with SMTP access

### Installation Steps

1. Prepare Router

# Connect to router
ssh root@192.168.1.1

# Install required packages (if needed)
opkg update
opkg install iptables-mod-extra curl

# Create directories
mkdir -p /etc/firewall
mkdir -p /usr/bin
mkdir -p /www/cgi-bin

2. Deploy Router Files

# From host machine
scp router/etc/firewall.captive root@router-ip:/etc/
scp router/etc/rc.local root@router-ip:/etc/
scp router/usr/bin/captive-auth root@router-ip:/usr/bin/
scp -r router/www/* root@router-ip:/www/

# Set permissions
ssh root@router-ip "chmod +x /etc/firewall.captive"
ssh root@router-ip "chmod +x /usr/bin/captive-auth"
ssh root@router-ip "chmod +x /www/cgi-bin/*"

3. Configure Network Interfaces

Edit /etc/config/network on router:

config interface 'lan'
    option device 'eth1'
    option proto 'static'
    option ipaddr '10.0.10.1'
    option netmask '255.255.255.0'

config interface 'hostbridge'
    option device 'eth2'
    option proto 'static'
    option ipaddr '192.168.56.2'
    option netmask '255.255.255.0'

4. Setup OTP Server

# Install dependencies
pip3 install flask flask-cors requests

# Configure email settings
nano otp_auth_server.py
# Edit SMTP settings:
# EMAIL_ENABLED = True
# SMTP_SERVER = "your-smtp-server"
# SMTP_USERNAME = "your-email@example.com"
# SMTP_PASSWORD = "your-password"

# Test email configuration
python3 test_email.py

# Run server
python3 otp_auth_server.py

5. Start Captive Portal

# On router
/etc/firewall.captive

# Or reboot to auto-start
reboot

6. Verify Operation

# Check firewall chains
iptables -L CAPTIVE_PORTAL -n -v
iptables -L CAPTIVE_ACCEPT -n -v

# Check NAT rules
iptables -t nat -L PREROUTING -n

# Check DNS configuration
cat /etc/dnsmasq.conf | grep captive

# View logs
logread | grep captive

## Testing and Validation

### Test Cases

1. Captive Portal Detection

| Platform | Test | Expected Result | Status |
| --- | --- | --- | --- |
| iOS 17+ | Connect to WiFi | Portal popup appears | ✅ Pass |
| Android 14+ | Connect to WiFi | Portal notification | ✅ Pass |
| macOS | Connect to WiFi | Portal popup | ✅ Pass |
| Windows 11 | Connect to WiFi | Portal page opens | ✅ Pass |
| Linux | Browse HTTP site | Redirect to portal | ✅ Pass |

2. Authentication Flow

| Test Case | Expected Behavior | Status |
| --- | --- | --- |
| Valid email entry | OTP sent to email | ✅ Pass |
| Invalid email format | Error message displayed | ✅ Pass |
| Valid OTP entry | Authentication successful | ✅ Pass |
| Invalid OTP | Error message shown | ✅ Pass |
| Expired OTP | Error with re-request option | ✅ Pass |
| Used OTP resubmission | Error message | ✅ Pass |

3. Firewall Behavior

| Scenario | Expected Result | Status |
| --- | --- | --- |
| Unauthenticated HTTP | Redirect to portal | ✅ Pass |
| Unauthenticated HTTPS | Connection rejected | ✅ Pass |
| Unauthenticated DNS | Only router DNS works | ✅ Pass |
| Authenticated traffic | Full internet access | ✅ Pass |
| Session expiration | Access revoked | ✅ Pass |

4. Edge Cases

- Multiple devices same email: ✅ Each device gets unique session

- Concurrent OTP requests: ✅ Latest OTP invalidates previous

- Router reboot: ✅ Firewall rules restored, sessions cleared

- OTP server offline: ✅ Graceful error message

- Email delivery failure: ✅ User notified to try again

## Performance Metrics

### Server Performance

- OTP Generation Time: <10ms

- Email Delivery: 1-3 seconds (depends on SMTP)

- OTP Verification: <5ms

- Session Lookup: O(1) - hash table

- Memory Usage: ~50MB (Flask + Python)

- Concurrent Users: Tested up to 50 simultaneous

### Router Performance

- iptables Rule Check: <1ms per packet

- NAT Translation: <1ms

- CGI Response Time: 10-20ms

- DNS Query: 1-5ms

- Memory Usage: ~20MB additional

- CPU Usage: <5% on average

### Network Latency

- Portal Detection: 0.5-2 seconds

- Splash Page Load: <500ms

- API Request: 50-200ms

- Total Auth Time: 5-15 seconds (user-dependent)

## Troubleshooting Guide

### Common Issues

Issue: Portal Not Detected

Symptoms: Devices connect but no popup appears

Solutions: 1. Check DNS configuration: cat /etc/dnsmasq.conf 2. Verify detection URLs configured 3. Test DNS: nslookup captive.apple.com 10.0.10.1 4. Check DHCP options: cat /tmp/dhcp.leases

Issue: OTP Email Not Received

Symptoms: Email form succeeds but no email arrives

Solutions: 1. Check SMTP credentials in otp_auth_server.py 2. Test email: python3 test_email.py 3. Check spam/junk folder 4. Verify EMAIL_ENABLED = True 5. Check server logs: journalctl -f

Issue: OTP Verification Fails

Symptoms: Valid OTP rejected

Solutions: 1. Check OTP expiration (5 minutes) 2. Ensure OTP not already used 3. Verify server time synchronized 4. Check server logs for details 5. Test MAC detection: curl http://10.0.10.1/cgi-bin/get-mac

Issue: No Internet After Auth

Symptoms: OTP accepted but still no internet

Solutions: 1. Check iptables rules: iptables -L CAPTIVE_ACCEPT -n -v 2. Verify MAC in accept chain 3. Check NAT RETURN rule: iptables -t nat -L PREROUTING -n 4. Test router auth: /usr/bin/captive-auth list 5. Check router connectivity to WAN

Issue: Firewall Rules Not Applying

Symptoms: Direct internet access without auth

Solutions: 1. Check if firewall script ran: logread | grep captive 2. Manually run: /etc/firewall.captive 3. Verify rc.local executable: chmod +x /etc/rc.local 4. Check interface names match (eth1) 5. Reboot router

## Monitoring and Logging

### Server Logs

Flask Application:

# View real-time logs
python3 otp_auth_server.py

# Sample output:
[21:05:32] 📧 OTP 123456 requested for user@example.com
[21:06:15] ✅ Authenticated: AA:BB:CC:DD:EE:FF (user@example.com) with OTP 123456
[21:06:15] 🔓 Authenticating AA:BB:CC:DD:EE:FF on router...
[21:06:15] ✅ Router auth successful: AA:BB:CC:DD:EE:FF

Admin Dashboard:

Access: http://192.168.56.1:5000

Displays:
- Active OTPs count
- Authenticated clients count
- Pending registrations
- Total OTPs generated
- Recent OTP requests (table)
- Authenticated clients (table)
- Auto-refresh every 30 seconds

### Router Logs

System Log:

# View captive portal logs
logread | grep captive

# Sample output:
captive-firewall: Setting up captive portal firewall rules...
captive-firewall: Created CAPTIVE_ACCEPT chain
captive-firewall: Created CAPTIVE_DNS chain
captive-firewall: Created CAPTIVE_PORTAL chain
captive-portal: Authenticated client AA:BB:CC:DD:EE:FF (10.0.10.50)

View Authentication State:

# List authenticated clients
/usr/bin/captive-auth list

# Output:
=== Authenticated clients ===
Chain CAPTIVE_ACCEPT (1 references)
pkts bytes target prot opt in out source destination
 1234 567K ACCEPT all -- * * 0.0.0.0/0 0.0.0.0/0 MAC AA:BB:CC:DD:EE:FF

=== NAT Rules ===
num target prot opt source destination
1 RETURN tcp -- 0.0.0.0/0 0.0.0.0/0 tcp dpt:80 MAC AA:BB:CC:DD:EE:FF

## Future Enhancements

### Planned Features

1. SMS OTP Option - Integrate Twilio or similar service - Fallback for users without email - Faster delivery than email

2. Rate Limiting - Limit OTP requests per email/IP - Prevent abuse and flooding - Configurable thresholds

3. Database Backend - Replace in-memory storage with SQLite/PostgreSQL - Persistent sessions across server restarts - Historical analytics and reporting

4. HTTPS Support - SSL/TLS certificate on router - Encrypted portal communication - Let’s Encrypt integration

5. Multi-Language Support - Internationalization (i18n) - Language detection from browser - Translate splash page and emails

6. Advanced Analytics - Usage statistics and graphs - Peak usage times - User demographics - Connection duration tracking

7. Social Login - OAuth integration (Google, Facebook) - Alternative to email OTP - Faster authentication

8. Mobile App - Dedicated iOS/Android app - Push notifications for OTP - QR code authentication

## Conclusion

This captive portal implementation successfully combines modern web technologies (Flask, JavaScript) with traditional network security tools (iptables, dnsmasq) to create a robust, user-friendly WiFi authentication system.

### Key Achievements

✅ RFC 8910 Compliance - Standards-based captive portal detection ✅ Multi-Platform Support - Works on all major operating systems ✅ Secure Architecture - Defense-in-depth with multiple security layers ✅ User-Friendly - Modern UI with step-by-step guidance ✅ Scalable - Handles multiple concurrent users efficiently ✅ Maintainable - Clean code with clear separation of concerns ✅ Documented - Comprehensive documentation and comments

### Lessons Learned

- Selective DNS hijacking is superior to total DNS interception

- iptables chain organization is crucial for maintainability

- MAC address binding provides good balance of security and usability

- Email OTP is widely accessible but has delivery delays

- Browser detection methods vary significantly across platforms

### Production Readiness

This system is suitable for: - ✅ Small to medium deployments (1-100 users) - ✅ Educational environments - ✅ Guest WiFi in offices/cafes - ✅ Home networks with guest access - ⚠️ Enterprise (needs database, redundancy, monitoring) - ⚠️ High-security (needs additional authentication factors)

## Appendix

### A. File Structure

captive_portal_project/
├── otp_auth_server.py           # Main authentication server
├── otp_auth_server_adapted.py   # Adapted version
├── splash_otp.html               # Splash page (development)
├── test_email.py                 # Email testing utility
├── README.md                     # Setup documentation
└── router/                       # Router configuration
    ├── etc/
    │   ├── firewall.captive      # Main firewall script
    │   └── rc.local              # Startup script
    ├── usr/
    │   └── bin/
    │       └── captive-auth      # Auth management script
    └── www/
        ├── simple-otp.html       # Router splash page
        ├── 404.html              # Error page
        └── cgi-bin/
            ├── auth              # Auth endpoint
            ├── api-proxy         # API proxy
            ├── captive-detect    # Detection handler
            └── get-mac           # MAC detection

### B. Port Reference

| Port | Service | Purpose |
| --- | --- | --- |
| 80 | HTTP | Captive portal redirect |
| 443 | HTTPS | Blocked (forces detection) |
| 53 | DNS | Selective hijacking |
| 5000 | Flask | OTP server API |
| 8080 | Proxy | Client → OTP server |
| 587 | SMTP | Email delivery (TLS) |

### C. IP Address Scheme

| Network | Address | Purpose |
| --- | --- | --- |
| 10.0.10.0/24 | 10.0.10.1 | Router LAN |
| 10.0.10.0/24 | 10.0.10.2-254 | DHCP clients |
| 192.168.56.0/24 | 192.168.56.1 | Host machine |
| 192.168.56.0/24 | 192.168.56.2 | Router bridge |

### D. API Reference

Request OTP

POST /api/request_otp HTTP/1.1
Content-Type: application/json

{
  "email": "user@example.com"
}

Response:
{
  "success": true,
  "message": "OTP sent to your email",
  "validity": 300
}

Verify OTP

POST /api/verify_otp HTTP/1.1
Content-Type: application/json

{
  "otp": "123456",
  "mac": "AA:BB:CC:DD:EE:FF"
}

Response:
{
  "success": true,
  "token": "xxx",
  "expires_in": 3600,
  "message": "Authentication successful",
  "router_auth": true
}

Check Authentication

GET /api/check_auth?mac=AA:BB:CC:DD:EE:FF HTTP/1.1

Response:
{
  "authenticated": true,
  "email": "user@example.com",
  "expires_in": 2500
}

### E. iptables Chain Reference

View All Chains

iptables -L -n -v
iptables -t nat -L -n -v

CAPTIVE_PORTAL Chain

iptables -L CAPTIVE_PORTAL -n -v --line-numbers

CAPTIVE_ACCEPT Chain

iptables -L CAPTIVE_ACCEPT -n -v

NAT PREROUTING

iptables -t nat -L PREROUTING -n --line-numbers

### F. Configuration Templates

dnsmasq.conf additions:

# Captive Portal Detection
address=/captive.apple.com/10.0.10.1
address=/connectivitycheck.gstatic.com/10.0.10.1
address=/detectportal.firefox.com/10.0.10.1
address=/www.msftconnecttest.com/10.0.10.1
address=/clients3.google.com/10.0.10.1

# DHCP Options
dhcp-option=114,http://10.0.10.1/simple-otp.html
dhcp-option=160,http://10.0.10.1/cgi-bin/captive-detect

End of Report

For questions or support, please refer to the GitHub repository or contact the development team.